dB Meter + Audio Visualizer Playbook

## I. Challenge

Develop a mixed-signal Digital Signal Processing (DSP) pipeline that captures real-time acoustic waveforms, transforms time-domain data into frequency spectra, and computes relative sound pressure levels in decibels (dB). The system must cleanly separate sensitive analog sensor signals from high-current digital switching components while mapping volume parameters to a responsive, dynamic RGB visualizer ring.

## II. Guide

### A. Concept

To process real-time audio and drive an addressable LED grid smoothly, we rely on these core embedded and DSP principles:

- 1. High-Speed Fixed-Interval Acquisition: To accurately capture audio waves, an Analog-to-Digital Converter (ADC) must sample the incoming signal at a uniform frequency. To resolve frequencies up to a certain limit, the sampling frequency must double that target (known as the Nyquist boundary).

  - Learn more: Understanding the Nyquist-Shannon Sampling Theorem

- 2. Dynamic DC Offset Cancellation: Electret microphones ride on a constant baseline DC voltage reference. To isolate true sound waves, the software calculates the arithmetic mean across a batch of samples and subtracts it from each reading, centering the waveform neatly at zero.

  - Learn more: DC Bias and Signal Conditioning in AC Audio Circuits

- 3. Root-Mean-Square (RMS) & Logarithmic Scale Mapping: Isolated background noise spikes can skew simple peak measurements. Computing the Root-Mean-Square (VRMS) yields the true continuous electrical power of the wave, which is then scaled logarithmically to match human auditory perception:

dB=20×log10VRMS+0.1+Offsetcalibration

  - Learn more: Measuring Sound and Audio Signals in Decibels

- 4. Fast Fourier Transform (FFT) Resolution: To view the audio by its individual pitch bands, time-domain data is run through a radix-2 FFT algorithm after applying a Hamming window to suppress boundary edge leaks. The width of each frequency bin depends on the sample count (N) and the sampling frequency (fs):

Δf=fsN

  - Learn more: The Fourier Transform Explained

- 5. HSV Color Space Transitions: Standard RGB manipulation makes creating smooth color flows difficult. By driving the visualizer ring using Hue-Saturation-Value (HSV) dynamics instead, the code can fluidly slide the color phase space around the spectrum purely based on audio intensity.

  - Learn more: Working with NeoPixel RGB LEDs

Real-World Example: Professional studio VU meters, graphic equalizers on consumer audio receivers, and smart voice assistants (like Amazon Echo or Google Nest). These systems process microphone arrays, apply noise-canceling software filters, and light up reactive LED rings to show voice tracking or volume metrics.

### B. Components

- Microcontroller: WeAct Studio ESP32-C6 (32-bit RISC-V architecture).

- Acoustic Transducer: LM393 Electret Microphone Module.

- Visual Interface: 16-element WS2812B addressable NeoPixel RGB LED Ring.

- Hardware Filtering: A 10μF to 100μF electrolytic capacitor (used to eliminate breadboard power rail noise).

### C. Connections

Because addressable LEDs pull rapid current surges that can corrupt sensitive analog readings, the power and data configurations must be strictly managed:

| Source Module Pin | Target MCU Pin (ESP32-C6) | Operational Description | Circuit Requirements / Notes |
| --- | --- | --- | --- |
| LM393 Mic VCC | V5 / VIN (+5V) | Main Sensor Power Input | Connect to +5V to enforce optimal electret bias and signal swing. |
| LM393 Mic GND | GND | Ground Reference | Shared ground plane. |
| LM393 Mic OUT | GPIO 1 | Time-Domain Analog Data | Keep this signal path as short as possible to avoid noise pickup. |
| NeoPixel 5V | V5 / VIN (+5V) | High-Current Power Rail | Place a decoupling capacitor (10μF–100μF) across Mic VCC and GND. |
| NeoPixel GND | GND | Ground Reference | Shared ground plane. |
| NeoPixel DI | GPIO 18 | High-Speed Control Stream | Delivers the 800 kHz digital data stream to update the pixels. |

### D. Code

This implementation features data acquisition timing locks, automatic DC offset removal, logarithmic decibel scaling, and an interactive HSV lighting renderer:

1. C++

2. /**

3.  * @file Audio_Processor_Visualizer.ino

4.  * @brief Integrated Audio DSP Pipeline (RMS dB, FFT, and HSV Rainbow Visualization)

5.  * @hardware WeAct Studio ESP32-C6, LM393 Microphone Module, 16-LED WS2812B Ring

6.  */

7.

8. #include <Adafruit_NeoPixel.h>

9. #include "arduinoFFT.h"

10.

11. // Hardware Pin Configuration

12. #define MIC_PIN         1

13. #define LED_PIN         18

14. #define NUM_LEDS        16

15.

16. // DSP Sampling Parameters

17. #define SAMPLES         128          // Must be a power of 2 (Radix-2 FFT requirement)

18. #define SAMPLING_FREQ   10000        // 10kHz Sample Frequency (Nyquist boundary = 5kHz)

19. double vReal[SAMPLES];

20. double vImag[SAMPLES];

21. unsigned long sampling_period_us;

22.

23. // Field-Calibrated Tuning Parameters

24. const int NOISE_FLOOR  = 4;          // Squelch value to eliminate background static

25. const int MANUAL_GAIN  = 4;          // Linear multiplier for high-impedance mic signal

26. const int MAX_EXPECTED = 1000;       // Upper amplitude ceiling for 100% LED fill

27.

28. // Calibrated Decibel Reference Parameters

29. const float DB_FLOOR   = 35.0;       // Lower limit for LED initiation (dB)

30. const float DB_CEILING = 75.0;       // Upper limit for full LED ring saturation (dB)

31. const float DB_OFFSET  = 40.0;       // Scale alignment factor to approximate standard meters

32.

33. // Graphic State Variables

34. uint16_t globalHue = 0;

35. const int HUE_SPEED = 400;           // Phase shift increment per audio frame

36.

37. // Device Instantiations

38. Adafruit_NeoPixel ring(NUM_LEDS, LED_PIN, NEO_GRB + NEO_KHZ800);

39. ArduinoFFT<double> FFT = ArduinoFFT<double>(vReal, vImag, SAMPLES, SAMPLING_FREQ);

40.

41. void setup() {

42.   Serial.begin(115200);

43.

44.   ring.begin();

45.   ring.setBrightness(40);            // Maintains power rail current limits

46.   ring.show();

47.

48.   analogReadResolution(12);          // Configures ESP32-C6 SAR ADC to 12-bit depth (0 - 4095)

49.   sampling_period_us = round(1000000 * (1.0 / SAMPLING_FREQ)); // Evaluates to 100 microseconds

50. }

51.

52. void loop() {

53.   double sumSquare = 0;

54.   double sampleSum = 0;

55.

56.   // 1. DATA ACQUISITION BLOCK (Strict Time-Domain Interval Sampling)

57.   for (int i = 0; i < SAMPLES; i++) {

58.     unsigned long microseconds = micros();

59.

60.     int raw = analogRead(MIC_PIN);

61.     vReal[i] = (double)raw;

62.     vImag[i] = 0;                    // Imaginary components initialized to zero

63.     sampleSum += raw;

64.

65.     // Enforce uniform sampling frequency interval

66.     while (micros() < (microseconds + sampling_period_us)) { /* Active Wait Lock */ }

67.   }

68.

69.   // 2. SIGNAL CONDITIONING & TIME-DOMAIN ANALYSIS (RMS & dB)

70.   double mean = sampleSum / SAMPLES;

71.   for (int i = 0; i < SAMPLES; i++) {

72.     double centered = vReal[i] - mean; // Dynamic DC offset cancellation

73.     sumSquare += (centered * centered);

74.     vReal[i] = centered;               // Stores pre-filtered data back into FFT array

75.   }

76.

77.   double rms = sqrt(sumSquare / SAMPLES);

78.   double dB = (20 * log10(rms + 0.1)) + DB_OFFSET; // Conversion to log scale

79.

80.   // 3. LEGACY VOLUMETRIC SQUELCH PIPELINE (LED Optimization Mapping)

81.   double signalMax = -9999;

82.   double signalMin = 9999;

83.   for (int i = 0; i < SAMPLES; i++) {

84.     if (vReal[i] > signalMax) signalMax = vReal[i];

85.     if (vReal[i] < signalMin) signalMin = vReal[i];

86.   }

87.   int rawSwing = (int)(signalMax - signalMin);

88.   int cleanSwing = rawSwing - NOISE_FLOOR;

89.   if (cleanSwing < 0) cleanSwing = 0;

90.   int amplifiedSwing = cleanSwing * MANUAL_GAIN;

91.

92.   // 4. FREQUENCY SPECTRUM PROCESSING (FFT Execution)

93.   FFT.windowing(FFT_WIN_TYP_HAMMING, FFT_FORWARD);

94.   FFT.compute(FFT_FORWARD);

95.   FFT.complexToMagnitude();

96.

97.   // 5. DIAGNOSTIC SERIAL TELEMETRY STREAM

98.   Serial.print("Total_dB:"); Serial.print(dB); Serial.print(",");

99.   Serial.print("Vol_Bar:");  Serial.print(amplifiedSwing); Serial.print(",");

100.   Serial.print("Bass_Band:"); Serial.print(20 * log10(vReal[2] + 0.1) + DB_OFFSET); Serial.print(",");

101.   Serial.print("Mid_Band:");  Serial.print(20 * log10(vReal[8] + 0.1) + DB_OFFSET); Serial.print(",");

102.   Serial.print("High_Band:"); Serial.println(20 * log10(vReal[32] + 0.1) + DB_OFFSET);

103.

104.   // 6. HARDWARE LEVEL RENDERING

105.   int displayLeds = map(amplifiedSwing, 0, MAX_EXPECTED, 0, NUM_LEDS);

106.   displayLeds = constrain(displayLeds, 0, NUM_LEDS);

107.

108.   if (displayLeds > 0) {

109.     globalHue += HUE_SPEED;

110.   }

111.

112.   for (int i = 0; i < NUM_LEDS; i++) {

113.     if (i < displayLeds) {

114.       uint16_t ledHue = globalHue + (i * 1000);

115.       ring.setPixelColor(i, ring.gamma32(ring.ColorHSV(ledHue, 255, 255)));

116.     } else {

117.       ring.setPixelColor(i, 0);

118.     }

119.   }

120.   ring.show();

121. }

122. III

123.

Student Tip: After successfully uploading the code using Arduino IDE, open the Serial Monitor (Ctrl+Shift+M or Tools > Serial Monitor) set to 115200 baud to view real-time numerical readings.

To see live visual graphs of the DSP calculations, close the Serial Monitor and open the Serial Plotter (Ctrl+Shift+L or Tools > Serial Plotter). Test your results by playing music on the mic module.

## III. Play & Share

- Build a Graphic Equalizer: Instead of driving the entire LED ring based on overall volume (amplifiedSwing), use the computed FFT array variables (vReal[i]). Divide the ring into distinct physical sectors mapped specifically to Bass, Mids, and Highs to create a true frequency visualizer!

- Develop a Peak Hold Indicator: Real audio hardware leaves a single LED lit up at the highest sound level achieved for a brief second before slowly falling back down. Try modifying the hardware loop to track a peakHoldIndex that stays floating above the active volume bars.

- Implement an Automatic Gain Controller (AGC): If you take the visualizer into a quiet room or a loud concert, the hardcoded limits (MANUAL_GAIN and MAX_EXPECTED) won't adapt well. Try building a software tracking loop that dynamically rescales your dynamic range boundaries based on the rolling minimum and maximum sound levels captured over the last 10 seconds.