Rolling Marble Simulation Playbook

I. Challenge

Design and execute an embedded Hardware-in-the-Loop (HIL) physics engine that models the dynamic kinematics of a virtual sphere rolling along a circular path. The system must translate continuous spatial data from an inertial sensor into dynamic physical properties—such as inertia, momentum, friction, and mass—and map the resulting high-precision floating-point coordinates onto a discrete 16-LED addressable display ring with zero latency at a steady 50 Hz execution loop.

II. Guide

A. Concept

To implement a responsive, realistic physics simulation on a microcontroller, we apply these key trigonometric and classical mechanics concepts:

- 1. Downhill Incline Resolver: Micro-electromechanical system (MEMS) accelerometers measure raw gravitational vectors along spatial axes. To isolate the true direction that gravity pulls a physical object downwards, the system relies on the two-argument arctangent function:

θtarget=atan2xAngle,yAngle×180π

This evaluates the quadrant signs to map the absolute tilt target from 0∘ to 360∘.

  - Learn more: The Mathematics of the Atan2 Function

- Modular Angular Distance & Wrap-Around: Direct subtraction (θtarget-θmarble) breaks down across the 0∘/360∘ boundary, causing false rotational directions. The physics engine constrains angular error to -180∘,180∘ to ensure the simulated marble always takes the shortest physical path around the circular track.

  - Learn More: Stack Overflow – Finding the Shortest Distance Between Two Angles

- 3. Semi-Implicit Euler Numerical Integration: Instead of complex continuous equations, the physics engine updates the particle across tiny discrete slices of time (Δt=20 ms). Restoring torque yields acceleration, which modifies the current velocity before being compressed by a damping factor (μ) representing surface friction:

vt=vt-1+a×μ

xt=xt-1+vt

  - Learn more: Numerical Integration

- 4. Continuous-to-Discrete Remapping: The physics core operates entirely with high-precision floating-point variables to maintain momentum accuracy. To display this on hardware, the continuous angle must be scaled, rounded to the nearest integer index, and wrapped using a modulo operation to light the correct pixel:

IndexLED=roundθmarble360/NLEDs mod NLEDs

  - Learn more: Quantization Error Explained

Real-World Example: Advanced stability control systems in aerospace rocketry, smartphone screen orientation adjustments, and active anti-sway suspension structures in high-performance automotive platforms. They sample real-time tilt vectors to shift virtual or mechanical elements against gravity.

B. Components

- Microcontroller: WeAct Studio ESP32-C6 Dev Board.

- Motion Sensor: MPU6050 6-DoF Inertial Measurement Unit.

- Visualizer Array: 16-element WS2812B NeoPixel RGB LED Ring.

- Interconnect Base: Solderless breadboard and jumper hookup wires.

C. Connections

The system uses an I2C protocol channel to extract the sensor readings and a high-speed one-wire pulse control channel to update the lighting ring:

| Source Device | Pin Label | Target MCU Pin (ESP32-C6) | Operational Functional Description |
| --- | --- | --- | --- |
| MPU6050 IMU | VCC | 3.3V | Logic Power Supply |
|  | GND | Gnd | Common Ground Reference |
|  | SDA | GPIO 6 | Hardware I2C Serial Data Communication |
|  | SCL | GPIO 7 | Hardware I2C Serial Clock Line |
| NeoPixel Ring | 5V | V5 / VIN | High-Current Input (Direct USB 5V Rail) |
|  | GND | Gnd | Common Ground Reference |
|  | DI | GPIO 18 | High-Speed Digital Pixel Control Input |
|  | DO | Unconnected | Open Daisy-Chain Output Pass-through |

D. Code

This program sets up the hardware lines, calibrates the gyroscope base offset, and runs the integration equations at a consistent 50 Hz step interval:

1. #include <Wire.h>

2. #include <MPU6050_tockn.h>

3. #include <Adafruit_NeoPixel.h>

4.

5. #define LED_PIN     18

6. #define NUM_LEDS    16

7. #define I2C_SDA     6

8. #define I2C_SCL     7

9.

10. MPU6050 mpu6050(Wire);

11. Adafruit_NeoPixel ring(NUM_LEDS, LED_PIN, NEO_GRB + NEO_KHZ800);

12.

13. // --- PHYSICS ENGINE VARIABLES ---

14. float marblePos = 0.0;       // Current position of the marble in degrees (0.0 to 360.0)

15. float marbleVelocity = 0.0;  // Current angular velocity

16. const float FRICTION = 0.92; // Damping factor (0.99 = ice/no friction, 0.80 = heavy mud)

17. const float GRAVITY_MULT = 0.02; // How responsive the marble is to gravity/weight

18.

19. void setup() {

20.   Serial.begin(115200);

21.   Wire.begin(I2C_SDA, I2C_SCL);

22.

23.   ring.begin();

24.   ring.setBrightness(40);

25.   ring.show();

26.

27.   Serial.println("Place flat for IMU calibration...");

28.   mpu6050.begin();

29.   mpu6050.calcGyroOffsets(true);

30.   Serial.println("Physics Engine Ready!");

31. }

32.

33. void loop() {

34.   mpu6050.update();

35.   float xAngle = mpu6050.getAngleX();

36.   float yAngle = mpu6050.getAngleY();

37.

38.   // 1. Calculate overall tilt steepness (Magnitude)

39.   float tiltMagnitude = sqrt((xAngle * xAngle) + (yAngle * yAngle));

40.

41.   // 2. Only update physics if the board is actually tilted

42.   if (tiltMagnitude > 2.0) {

43.     // Find the downhill target angle where gravity pulls a marble (opposite of bubble level)

44.     float targetAngle = atan2(xAngle, yAngle) * 180.0 / M_PI;

45.     if (targetAngle < 0) targetAngle += 360.0;

46.

47.     // 3. Calculate shortest angular distance to target (handles 0/360 wrap-around)

48.     float angularError = targetAngle - marblePos;

49.     if (angularError > 180.0)  angularError -= 360.0;

50.     if (angularError < -180.0) angularError += 360.0;

51.

52.     // 4. Euler Integration: Accel -> Velocity -> Position

53.     float acceleration = angularError * tiltMagnitude * GRAVITY_MULT;

54.

55.     marbleVelocity += acceleration;

56.     marbleVelocity *= FRICTION; // Apply simulated surface resistance

57.     marblePos += marbleVelocity;

58.

59.     // Keep marble position strictly within 0-360 degrees

60.     if (marblePos >= 360.0) marblePos -= 360.0;

61.     if (marblePos < 0.0)    marblePos += 360.0;

62.   } else {

63.     // If the board is perfectly flat, slowly bleed off any remaining velocity

64.     marbleVelocity *= 0.5;

65.     marblePos += marbleVelocity;

66.   }

67.

68.   // 5. Map the float position angle to the nearest integer LED index

69.   int currentLED = ((int)round(marblePos / (360.0 / NUM_LEDS))) % NUM_LEDS;

70.

71.   // 6. Render the Marble

72.   ring.clear();

73.

74.   // Draw the main marble (Bright Blue)

75.   ring.setPixelColor(currentLED, ring.Color(0, 150, 255));

76.

77.   // Draw a dim "tail" or motion blur behind the marble based on velocity direction

78.   int tailLED = (marbleVelocity > 0) ? (currentLED - 1 + NUM_LEDS) % NUM_LEDS : (currentLED + 1) % NUM_LEDS;

79.   if (abs(marbleVelocity) > 2.0) {

80.     ring.setPixelColor(tailLED, ring.Color(0, 40, 100));

81.   }

82.   ring.show();

83.

84.   delay(20); // Steady 50Hz frame rate for smooth physics integration

85. }

86.

III. Play & Share

- Introduce Elastic Boundary Collisions: Instead of allowing the marble to spin continuously all the way around the ring, create a virtual brick barrier at LED index 0 and 8. Modify your integration code so that if the marble's position hits a barrier, the position stops instantly, and the velocity multiplies by -0.6 to bounce backwards!

- Simulate Multi-Body Physics: Expand your array parameters to introduce a second marble onto the track. Can you write a software check that senses when both indexes share the same discrete space and performs an elastic momentum swap so the particles bump away from each other?

- Design a Dynamic Maze Game: Use the code to build a game where a player tilts the board to guide the marble into a designated, blinking "target gate" LED within a specific time limit while avoiding dynamic, red "trap" LEDs that reset the score.