# Morse Code Decoder Playbook

## I. Challenge

Build a non-blocking embedded communication decoder that translates real-time manual dot/dash inputs into decoded alphanumeric text. The system must process debounced digital switch state changes, manage asynchronous timing windows (millis()) for dynamic character and word boundary isolation, provide instantaneous tactile/audio feedback, and render active output buffers on an SPI OLED display with zero processor stalling.

## II. Guide

### A. Concept

#### What is Morse Code?

Morse Code is a character encoding scheme designed in the 1830s for long-distance telecommunication via electric telegraph lines. It translates standard letters, numbers, and punctuation into rhythmic sequences of two discrete signal durations:

- Dot (⋅): The short signal duration, serving as the fundamental unit of time measurement (1 unit).

- Dash (-): The long signal duration, equal in length to exactly three dots (3 units).

In traditional telegraphy, timing intervals between signals define words and letters: spaces between dots/dashes within a single character equal 1 unit, character gaps equal 3 units, and word spaces equal 7 units. In this embedded project, software timers (LETTER_TIMEOUT and WORD_TIMEOUT) automatically group dot/dash inputs and convert them into standard ASCII characters.

#### i. Underlying Engineering Principles & Reference Links

- Asynchronous Non-Blocking Event Loops (millis() Timing): Using rigid delay() functions freezes microcontrollers and misses subsequent switch presses. Polling timestamps via millis() allows the system to continuously check inputs while evaluating background timers for letter timeouts (1000 ms) and word spaces (3000 ms).

  - Learn More: Arduino Documentation – Using millis() for Non-Blocking Timing

- State-Change Edge Detection: Measuring continuous button levels triggers repeated inputs every millisecond a switch remains depressed. Edge detection tracks transitioning states (HIGH→LOW), capturing exactly one discrete event per physical button press.

  - Learn More: Arduino Documentation – State Change Detection (Edge Detection)

- Contact Switch Debouncing: Mechanical switch contacts physically bounce upon actuation, causing microsecond-level noise spikes that register as multiple false presses. Combining hardware pull-up topologies with software debounce intervals stabilizes the logic level before registering an event.

  - Learn More: Digi-Key – Guide to Switch Debouncing Principles

- Binary Tree & Dictionary Parsing: Morse code maps variable-length symbol sequences (⋅ and -) directly to fixed alphanumeric ASCII characters. Looking up encoded buffer strings against a dictionary array converts time-domain signals into human-readable text.

  - Learn More: GeeksforGeeks – Binary Search Trees and Lookup Structures

- High-Speed Bus Communication (Hardware SPI): Driving graphic displays via bit-banging consumes substantial processor cycles. Utilizing dedicated hardware Serial Peripheral Interface (SPI) buses offloads frame updates to high-speed shift registers.

  - Learn More: SparkFun – Serial Peripheral Interface (SPI) Architecture

#### ii. Real-World Example

Aviation emergency radio beacons, naval signal lamps, telegraphy decoders, and accessibility switch interfaces (such as assistive sip-and-puff communication pads for motor-impaired individuals). These platforms translate discrete pulse duration patterns into digital text streams.

### B. Components

- Microcontroller: WeAct Studio ESP32-C6 Dev Board

- Display Interface: 0.96-inch SPI OLED Display Module (SSD1306 driver)

- Audio Transducer: Active Buzzer Module (Active HIGH)

- User Input: 2× Push Button Switches (Active LOW with internal pull-up)

- Interconnect Base: Solderless breadboard and jumper wires

### C. Connections

The system operates entirely on 3.3V logic. Input push buttons utilize internal MCU pull-up resistors (INPUT_PULLUP), pulling the pin to GND (0V) upon activation.

| Peripheral Component | Component Pin | ESP32-C6 Pin | Function / Designation |
| --- | --- | --- | --- |
| SPI OLED | GND | GND | Common Ground Plane |
| SPI OLED | VCC | 3.3V | Logic Power Supply |
| SPI OLED | SCLK (D0) | GPIO 6 | Hardware SPI Clock Line |
| SPI OLED | MOSI (D1) | GPIO 7 | Hardware SPI MOSI Data Line |
| SPI OLED | RES (RST) | GPIO 11 | Hardware Reset Line |
| SPI OLED | DC (A0) | GPIO 9 | Data/Command Mode Select |
| SPI OLED | CS | GPIO 10 | Active-Low Peripheral Chip Select |
| Dot Input Button | OUT / Signal | GPIO 1 | Debounced Dot State-Change Input |
| Dash Input Button | OUT / Signal | GPIO 2 | Debounced Dash State-Change Input |
| Active Buzzer | I/O / Signal | GPIO 3 | Audio Actuation Output (Active HIGH) |

### D. Code

1. C++

2. #include <SPI.h>

3. #include <Adafruit_GFX.h>

4. #include <Adafruit_SSD1306.h>

5.

6. // --- Display Configuration ---

7. #define SCREEN_WIDTH 128

8. #define SCREEN_HEIGHT 64

9.

10. #define OLED_MOSI   7

11. #define OLED_CLK    6

12. #define OLED_DC     9

13. #define OLED_CS    10

14. #define OLED_RESET 11

15.

16. Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, OLED_MOSI, OLED_CLK, OLED_DC, OLED_RESET, OLED_CS);

17.

18. // --- GPIO Definitions ---

19. const int DOT_BUTTON  = 1;

20. const int DASH_BUTTON = 2;

21. const int BUZZER      = 3;

22.

23. // --- Timing & State Variables ---

24. const unsigned long LETTER_TIMEOUT = 1000;

25. const unsigned long WORD_TIMEOUT   = 3000;

26. const int AUDIO_FEEDBACK_MS = 80;

27.

28. String morseBuffer = "";

29. String decodedText = "";

30. unsigned long lastInputTime = 0;

31.

32. bool letterPending = false;

33. bool wordPending = false;

34.

35. int lastDotState = HIGH;

36. int lastDashState = HIGH;

37.

38. // --- Translation Logic ---

39. char translateMorse(String morse) {

40.   if (morse == ".-")   return 'A'; if (morse == "-...") return 'B';

41.   if (morse == "-.-.") return 'C'; if (morse == "-..")  return 'D';

42.   if (morse == ".")    return 'E'; if (morse == "..-.") return 'F';

43.   if (morse == "--.")  return 'G'; if (morse == "....") return 'H';

44.   if (morse == "..")   return 'I'; if (morse == ".---") return 'J';

45.   if (morse == "-.-")  return 'K'; if (morse == ".-..") return 'L';

46.   if (morse == "--")   return 'M'; if (morse == "-.")   return 'N';

47.   if (morse == "---")  return 'O'; if (morse == ".--.") return 'P';

48.   if (morse == "--.-") return 'Q'; if (morse == ".-.")  return 'R';

49.   if (morse == "...")  return 'S'; if (morse == "-")    return 'T';

50.   if (morse == "..-")  return 'U'; if (morse == "...-") return 'V';

51.   if (morse == ".--")  return 'W'; if (morse == "-..-") return 'X';

52.   if (morse == "-.--") return 'Y'; if (morse == "--..") return 'Z';

53.

54.   if (morse == "-----") return '0'; if (morse == ".----") return '1';

55.   if (morse == "..---") return '2'; if (morse == "...--") return '3';

56.   if (morse == "....-") return '4'; if (morse == ".....") return '5';

57.   if (morse == "-....") return '6'; if (morse == "--...") return '7';

58.   if (morse == "---..") return '8'; if (morse == "----.") return '9';

59.

60.   return '?';

61. }

62.

63. // --- Subroutines ---

64. void updateDisplay() {

65.   display.clearDisplay();

66.

67.   display.setTextSize(2);

68.   display.setTextColor(SSD1306_WHITE);

69.   display.setCursor(0, 0);

70.   display.print("Buf:");

71.   display.print(morseBuffer);

72.

73.   display.drawFastHLine(0, 24, 128, SSD1306_WHITE);

74.

75.   display.setTextSize(1);

76.   display.setCursor(0, 32);

77.   display.print("Text:");

78.   display.setCursor(0, 45);

79.   display.setTextSize(2);

80.

81.   // Truncate output to prevent visual overflow

82.   if (decodedText.length() > 10) {

83.     display.print(decodedText.substring(decodedText.length() - 10));

84.   } else {

85.     display.print(decodedText);

86.   }

87.

88.   display.display();

89. }

90.

91. void triggerBuzzer() {

92.   digitalWrite(BUZZER, HIGH);

93.   delay(AUDIO_FEEDBACK_MS);

94.   digitalWrite(BUZZER, LOW);

95. }

96.

97. // --- Initialization ---

98. void setup() {

99.   Serial.begin(115200);

100.

101.   pinMode(DOT_BUTTON, INPUT_PULLUP);

102.   pinMode(DASH_BUTTON, INPUT_PULLUP);

103.

104.   pinMode(BUZZER, OUTPUT);

105.   digitalWrite(BUZZER, LOW);

106.

107.   if(!display.begin(SSD1306_SWITCHCAPVCC)) {

108.     for(;;); // System halt on peripheral failure

109.   }

110.

111.   display.clearDisplay();

112.   display.setTextSize(1);

113.   display.setTextColor(SSD1306_WHITE);

114.   display.setCursor(10, 20);

115.   display.print("System Ready");

116.   display.display();

117.   delay(1500);

118.

119.   updateDisplay();

120.   lastInputTime = millis();

121. }

122.

123. // --- Main Execution Loop ---

124. void loop() {

125.   unsigned long currentTime = millis();

126.

127.   int currentDotState = digitalRead(DOT_BUTTON);

128.   int currentDashState = digitalRead(DASH_BUTTON);

129.

130.   // Event Trigger: Dot Input

131.   if (currentDotState == LOW && lastDotState == HIGH) {

132.     morseBuffer += ".";

133.     triggerBuzzer();

134.     lastInputTime = currentTime;

135.     letterPending = true;

136.     wordPending = true;

137.     updateDisplay();

138.     delay(40);

139.   }

140.   lastDotState = currentDotState;

141.

142.   // Event Trigger: Dash Input

143.   if (currentDashState == LOW && lastDashState == HIGH) {

144.     morseBuffer += "-";

145.     triggerBuzzer();

146.     lastInputTime = currentTime;

147.     letterPending = true;

148.     wordPending = true;

149.     updateDisplay();

150.     delay(40);

151.   }

152.   lastDashState = currentDashState;

153.

154.   // Async Execution: Letter Translation

155.   if (letterPending && (currentTime - lastInputTime >= LETTER_TIMEOUT)) {

156.     char newChar = translateMorse(morseBuffer);

157.     decodedText += newChar;

158.

159.     // Diagnostic Serial Telemetry

160.     Serial.print("Decoded_Char:"); Serial.print(newChar); Serial.print(",");

161.     Serial.print("Full_Text:"); Serial.println(decodedText);

162.

163.     morseBuffer = "";

164.     letterPending = false;

165.     updateDisplay();

166.   }

167.

168.   // Async Execution: Word Spacing

169.   if (wordPending && !letterPending && (currentTime - lastInputTime >= WORD_TIMEOUT)) {

170.     decodedText += " ";

171.     wordPending = false;

172.

173.     Serial.print("Event:"); Serial.println("WORD_SPACE_ADDED");

174.     updateDisplay();

175.   }

176.

177. }

💡 Instructor Note / Student Tip: Open the Serial Monitor (Ctrl+Shift+M) at 115200 baud to trace decoded characters and watch letter timeouts trigger in real time.

You can also open the Serial Plotter (Ctrl+Shift+L) to inspect timing delays visually. Watching telemetry events on the plotter will demonstrate how LETTER_TIMEOUT (1000 ms) and WORD_TIMEOUT (3000 ms) reset dynamic timers after every keystroke.

## III. Play & Share

- Single-Key Auto-Length Timing (Single iambic keyer): Transform the hardware setup from a dual-button configuration into a single key system. Write an algorithm that measures press duration using millis()—short taps (<200 ms) record a Dot, while holding the key longer (>200 ms) records a Dash.

- Implement Adaptive WPM Speed Tuning: Add a 10 kΩ potentiometer to an analog input pin. Scale its 12-bit ADC reading (0-4095) to dynamically adjust LETTER_TIMEOUT between 400 ms (expert fast operator) and 2000 ms (beginner slow operator).

- Morse Code Training Game: Reverse the application! Program the ESP32-C6 to display a random ASCII letter on the OLED, play its audio Morse signature through the buzzer, and time how quickly the student can correctly re-enter the matching Dot/Dash combination.